# Wrist-Puck Dual-Channel Haptic Node

Complete Build Guide

Canada-Sourced BOM · ESP32-C3 Firmware · 3D-Printable Enclosure

Author: Sean (Whitehorse, YT)

May 2026  |  Revision 1.0

## Table of Contents

- Project Overview

- Bill of Materials (BOM) — Canada Sourced

- Wiring Diagram

- Assembly Notes

- Testing Checklist

- JSON Motif Schema Reference

- Troubleshooting

- Future Enhancements

# 1. Project Overview

The Wrist-Puck is a compact, wrist-worn haptic feedback device that combines two distinct sensory channels into a single 68 mm puck form factor. It is designed for use in VR/AR research, accessibility interfaces, and experimental interaction design.

### Dual-Channel Architecture

- Texture Channel: A 10 mm coin-type Linear Resonant Actuator (LRA) driven by a TI DRV2605L haptic controller. This channel delivers fine vibrotactile patterns including clicks, buzzes, and textural feedback using the DRV2605L’s built-in 123-effect waveform library or real-time playback (RTP) mode.

- Weight-Shift Channel: An Adafruit SG51R sub-micro servo actuates a sliding brass mass (~8–10 g) along a 30 mm linear rail inside the enclosure. As the mass moves, the user perceives a shift in weight or tilt on the wrist, simulating directional force cues.

The device is controlled wirelessly via Wi-Fi/UDP using JSON-formatted haptic motifs. A Seeed Studio XIAO ESP32-C3 serves as the brain, providing Wi-Fi 802.11 b/g/n, BLE 5.0, USB-C connectivity, and built-in LiPo charging in a tiny 21×17.8 mm package.

### Key Specifications

| Parameter | Value |
| --- | --- |
| Puck diameter | 68 mm |
| Height | 22 mm |
| Weight (assembled, est.) | ~55 g |
| Wireless | Wi-Fi 802.11 b/g/n, BLE 5.0 |
| Protocol | UDP port 9000, JSON motifs |
| Battery | 3.7 V, 400–500 mAh LiPo |
| Runtime (est.) | 2–4 hours active use |
| Charging | USB-C via XIAO built-in charger (or TP4056 backup) |

# 2. Bill of Materials (BOM) — Canada Sourced

All components below are available from Canadian suppliers or Amazon.ca with domestic shipping. Prices are in Canadian Dollars (CAD) as of May 2026.

| # | Component | Description | Supplier | SKU / ASIN | Qty | Est. Price (CAD) | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ESP32-C3 Dev Board | Seeed Studio XIAO ESP32-C3, 21×17.8 mm, Wi-Fi+BLE, USB-C, LiPo charging built-in | Amazon.ca | ASIN: B0B94JZ2YF | 1 | ~$43.73 | Amazon’s Choice; ships free. Built-in LiPo charge circuit makes TP4056 optional. |
| 2 | Haptic Driver | DRV2605L I2C Haptic Motor Controller Breakout Board (3–5 V) | Amazon.ca | Search “DRV2605L breakout” (NUEAYMS or similar) | 1 | ~$13.81 | Generic breakout compatible with Adafruit library. Verify I2C pull-ups present on board. |
| 3 | 10 mm Coin LRA | 10 mm Linear Resonant Actuator, C10-100 type, ~175 Hz, 2.0 V rated | DigiKey.ca | C10-100 (Precision Microdrives) or equiv. | 1 | ~$8–12 | Substitute with any 8–12 mm coin LRA. Check resonant frequency for DRV2605L auto-cal. |
| 4 | Sub-Micro Servo | Adafruit SG51R, 5 g, 25×22×12 mm, 0.6 kg·cm torque, 4.8 V | Mouser.ca | Part #: 485-2201 | 1 | ~$8.63 | Not readily on Amazon.ca. Alt: PiShop.ca (~$12.95). Substitute: any 5–9 g micro servo. |
| 5 | LiPo Battery | 3.7 V 500 mAh Lithium Polymer, JST-PH 2.0 connector, ~35×25×5 mm | Amazon.ca | Search “EEMB 500mAh 602040 LiPo JST” | 1 | ~$20.00 | Verify JST polarity matches XIAO before connecting! XIAO uses JST-SH 1.0 mm natively; may need adapter or direct solder. |
| 6 | LiPo Charger Module | TP4056 USB-C 1A Charging Board with dual protection (DW01 + FS8205) | Amazon.ca | Search “TP4056 Type-C USB 3pcs” | 1 (from 3-pack) | ~$3.17 | Optional if using XIAO’s built-in charger. Useful as standalone charge board or backup. |
| 7 | JST Connector Kit | JST PH 2.0 mm Wire Connector Kit, 450 pcs, 2/3/4-pin housings + female headers | Amazon.ca | Search “ZYNCUE JST PH 2.0mm 450pcs” | 1 kit | ~$10.79 | For battery, servo, LRA, and inter-board wiring. Enough for multiple nodes. |
| 8 | Perfboard | Small prototype PCB board, ~20×15 mm or 30×20 mm | Amazon.ca | Search “mini perfboard assortment” | 1 | ~$1.50 | From assortment pack (~$10–12 for 30+ boards). Used for voltage divider and wire junction. |
| 9 | Resistors | 100 kΩ ×2 (battery voltage divider), 4.7 kΩ ×2 (I2C pull-ups, if not on DRV board) | Amazon.ca | From any resistor assortment kit | 1 kit | ~$0.50 | 1/4 W or 1/8 W through-hole. |
| 10 | Hookup Wire | 26–30 AWG silicone stranded wire, assorted colors | Amazon.ca | Search “30 AWG silicone wire kit” | 1 | ~$12.00 | Flexible silicone insulation preferred for wearable routing. |
| 11 | Sliding Mass | Brass or steel slug, ~8–10 g, ~5×5×12 mm (or M5 brass hex nut stacked) | Hardware store / Amazon.ca | Search “M5 brass nut” or machine a small slug | 1 | ~$2.00 | Rides in the 6 mm-wide rail slot alongside the servo. Attach to servo horn via short linkage arm. |
| 12 | M2 Hardware | M2×6 mm screws + M2 nuts (for mounting boards to enclosure posts) | Amazon.ca | Search “M2 screw nut assortment” | 4 sets | ~$0.50 | Nylon preferred to avoid shorts. |
| 13 | 3D Print Filament | PLA or PETG, any color, ~25 g per puck | User’s stock | N/A | ~25 g | ~$1.00 | 0.2 mm layer height, 20% infill recommended. |
| 14 | Wrist Strap | 22 mm watch band (NATO style or silicone sport band) | Amazon.ca | Search “22mm NATO watch strap” | 1 | ~$8–15 | Threads through the enclosure strap slots. |

Total Estimated Cost: ~$115–135 CAD per node Well under $150 budget target.

Cost-Saving Tip Prices checked May 2026. Amazon.ca prices fluctuate; some items ship via third-party sellers. The XIAO ESP32-C3 is the most expensive single component — if sourcing directly from Seeed Studio (seeedstudio.com), the board is ~$6.99 USD + shipping, which may save $20+ CAD.

# 3. Wiring Diagram

### Pin Connection Table

| Signal | XIAO ESP32-C3 Pin | Target | Wire Color (suggested) | Notes |
| --- | --- | --- | --- | --- |
| I2C SDA | GPIO6 (D4) | DRV2605L SDA | Blue | 4.7 kΩ pull-up to 3.3 V if needed |
| I2C SCL | GPIO7 (D5) | DRV2605L SCL | Yellow | 4.7 kΩ pull-up to 3.3 V if needed |
| Servo PWM | GPIO5 (D3) | SG51R Signal (Orange) | Orange | Direct connection |
| Battery ADC | GPIO2 (D0) | Voltage divider midpoint | Green | 100k/100k divider from VBAT |
| 3.3 V Power | 3V3 | DRV2605L VIN | Red | 3.3 V rail |
| 5 V Power | 5V (USB) | SG51R VCC (Red) | Red | Servo needs 4.8–5 V |
| GND | GND | All GND | Black | Common ground bus |
| LRA+ | — | DRV2605L OUT+ | White | DRV drives LRA directly |
| LRA– | — | DRV2605L OUT– | White | DRV drives LRA directly |
| Battery+ | BAT+ pad | LiPo + (via JST) | Red | Through protection circuit |
| Battery– | BAT– pad / GND | LiPo – | Black | Common ground |

### ASCII Wiring Schematic

+---------------------+
                    |   XIAO ESP32-C3     |
                    |                     |
    +---- GPIO6 ----| D4 (SDA)            |
    |  +- GPIO7 ----| D5 (SCL)            |
    |  |  + GPIO5 --| D3 (PWM)            |
    |  |  |  + GP2 -| D0 (ADC)            |
    |  |  |  |      |               3V3 --|----- DRV2605L VIN
    |  |  |  |      |                5V --|----- SG51R VCC (Red)
    |  |  |  |      |               GND --|--+-- Common GND bus
    |  |  |  |      |              BAT+ --|--+-- LiPo + (JST)
    |  |  |  |      +---------------------+  |
    |  |  |  |                               |
    |  |  |  |    +----------------+         |
    |  |  |  |    |   DRV2605L     |         |
    |  |  |  |    |   Breakout     |         |
    +--+--+--+----| SDA            |         |
    |  +--+--+----| SCL            |         |
    |  |  |  |    | VIN -----------|--- 3.3V |
    |  |  |  |    | GND -----------|--- GND  |
    |  |  |  |    | OUT+ ==========#=> LRA + |
    |  |  |  |    | OUT- ==========#=> LRA - |
    |  |  |  |    +----------------+         |
    |  |  |  |                               |
    |  |  |  |    +----------------+         |
    |  |  |  |    |    SG51R       |         |
    |  |  |  |    |    Servo       |         |
    |  |  +--+----| Signal (Org)   |         |
    |  |     |    | VCC (Red) -----|--- 5V   |
    |  |     |    | GND (Brn) -----|--- GND  |
    |  |     |    +----------------+         |
    |  |     |                               |
    |  |     |    +----------------+         |
    |  |     |    | Voltage Div    |         |
    |  |     +----| Midpoint       |         |
    |  |          | VBAT--100k--Mid--100k--GND
    |  |          +----------------+         |
    |  |                                     |
    |  |          +----------------+         |
    |  |          |   10mm LRA     |         |
    |  |          |   (coin)       |         |
    |  |          |   Connected    |         |
    |  |          |   to DRV OUT   |         |
    |  |          +----------------+         |
    |  |                                     |
    |  |          +----------------+         |
    |  |          |  LiPo 500mAh   |         |
    |  |          |  3.7V          |         |
    |  |          |  JST-PH 2.0    |---------+
    |  |          +----------------+
    +--+

### Power Notes

Important — Power Considerations • The XIAO ESP32-C3 has a built-in LiPo charge IC. Connect the LiPo directly to the BAT pads on the XIAO. • The servo draws from the 5 V USB rail. When on battery only, the 3.3 V output may not be sufficient for the servo — consider a small boost converter (3.3 V → 5 V) for battery-only operation, OR accept reduced servo torque at 3.3 V.

CRITICAL — JST Polarity Verify JST polarity before connecting any LiPo battery. The XIAO uses + on the left when the USB port faces you. Reversed polarity WILL destroy the board instantly.

# 4. Assembly Notes

### Tools Required

| Tool | Specification / Notes |
| --- | --- |
| Soldering iron | Fine tip, 300–350 °C |
| Solder | Lead-free, 0.5 mm preferred |
| Wire strippers | 30 AWG capable |
| Flush cutters | For trimming leads and wire |
| Small Phillips screwdriver | M2 size |
| 3D printer | FDM, 0.4 mm nozzle |
| Multimeter | Continuity, voltage, resistance |
| Hot glue gun | Optional, for securing LiPo |
| Needle-nose pliers | For bending linkage wire |
| Computer | With Arduino IDE 2.x installed |

## Step 1: Print the Enclosure

- Open wrist_puck_enclosure.scad in OpenSCAD.

- Set RENDER = "bottom" → Press F6 (full render) → Export as bottom_shell.stl.

- Set RENDER = "lid" → Press F6 → Export as lid.stl.

- Slice at 0.2 mm layer height, 20% infill, PLA or PETG, no supports needed.

- Print time: ~1.5 hours total for both parts.

- Test-fit snap: the lid should click into the bottom shell with light pressure.

## Step 2: Prepare the Perfboard Junction

- Cut perfboard to ~20×15 mm.

- Solder a 3-pin header row for the power bus (+3.3 V, +5 V, GND).

- Solder the 100k/100k voltage divider (VBAT → midpoint → GND).

- Add 4.7 kΩ I2C pull-up resistors if your DRV2605L breakout doesn’t have them.

- Run hookup wires from the junction to each component location (leave 40 mm leads).

## Step 3: Wire the DRV2605L Breakout

- Solder 4 wires to DRV2605L: VIN, GND, SDA, SCL.

- Solder the LRA motor leads to the OUT+/OUT– pads on the DRV2605L.

- Use JST connectors for detachable connections, or solder directly for a permanent build.

- Test I2C address detection before installing in enclosure (address should read 0x5A).

## Step 4: Wire the SG51R Servo

- The SG51R comes with a standard 3-pin servo connector (Brown = GND, Red = VCC, Orange = Signal).

- Trim wires to ~60 mm.

- Solder or crimp JST-PH connectors for clean routing.

- Attach the smallest servo horn (single-arm) — this will push the sliding mass.

## Step 5: Prepare the Sliding Mass Assembly

- Thread 2× M5 brass nuts onto a short piece of brass rod, or use a single brass slug (~8–10 g).

- Drill a 1 mm hole in the mass for a short linkage wire to the servo horn.

- Verify the mass slides freely in the 6 mm × 30 mm rail slot in the enclosure.

- Attach to the servo horn tip with a small Z-bend linkage wire.

## Step 6: Install Components in Enclosure

- LRA: Press-fit the LRA into the floor pocket (LRA face down toward skin side). Secure with a tiny drop of CA glue.

- LRA wiring: Route LRA wires through the floor channel up to the DRV2605L pocket.

- Servo: Seat the SG51R servo in its cavity. The servo ears should rest on the cavity ledges. Secure with M2 screws or friction fit.

- Sliding mass: Place the sliding mass in the rail slot. Connect the linkage to the servo horn.

- DRV2605L: Mount the DRV2605L breakout in its pocket. Use double-sided foam tape or M2 screws.

- XIAO: Seat the XIAO ESP32-C3 in its pocket with the USB-C port aligned to the wall cutout.

- Battery: Place the LiPo battery in its compartment. Secure with a small strip of foam tape. Do NOT compress the battery.

- TP4056 (optional): If using the TP4056 separately, mount it in the rear pocket with USB-C port aligned to the rear wall cutout.

- Perfboard: Place the perfboard junction on top of the battery compartment.

- Wire routing: Route all wires through the floor channels. Tuck excess wire neatly.

- Close: Snap the lid on. Verify the LRA window aligns with the LRA and the status LED window aligns with the XIAO LED.

## Step 7: Flash the Firmware

- Connect the XIAO to your computer via USB-C.

- In Arduino IDE: Install ESP32 board support via Board Manager (Espressif ESP32, version 3.x).

- Select board: XIAO_ESP32C3.

- Install libraries: ESP32Servo, ArduinoJson (v7), Wire (built-in).

- Open wrist_puck_haptic_node.ino.

- Edit WIFI_SSID and WIFI_PASS constants to match your 2.4 GHz network.

- Upload (hold BOOT button if needed during upload).

- Open Serial Monitor at 115200 baud to watch calibration output.

# 5. Testing Checklist

Work through each item in order. Mark pass/fail for each test. All items must pass before the device is considered operational.

### Pre-Power Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 1 | ☐ | Visual inspection: no solder bridges, no exposed wire touching adjacent pads |
| 2 | ☐ | Multimeter continuity: GND bus connected to all component grounds |
| 3 | ☐ | Multimeter continuity: no short between 3.3 V and GND |
| 4 | ☐ | Multimeter continuity: no short between 5 V and GND |
| 5 | ☐ | LiPo polarity verified against XIAO BAT pad markings |
| 6 | ☐ | Servo connector seated correctly (Brown = GND, not reversed) |

### Power-On Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 7 | ☐ | Connect USB-C — XIAO power LED illuminates |
| 8 | ☐ | Serial monitor shows boot banner and pin configuration |
| 9 | ☐ | I2C scan detects DRV2605L at address 0x5A |
| 10 | ☐ | DRV2605L status register reads non-0xFF value |
| 11 | ☐ | Battery voltage ADC reads reasonable value (3.3–4.2 V, or 0 V if no battery connected) |

### Calibration Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 12 | ☐ | DRV2605L auto-calibration completes without DIAG_RESULT error |
| 13 | ☐ | LRA produces audible/tactile buzz during calibration “strong click” test |
| 14 | ☐ | Servo sweep test executes: center → left → right → center (smooth motion) |
| 15 | ☐ | Sliding mass moves freely in rail without binding |
| 16 | ☐ | No unusual sounds (grinding, buzzing, rattling) during servo sweep |

### Wi-Fi Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 17 | ☐ | Serial shows “Connected” with valid IP address |
| 18 | ☐ | RSSI reported (should be better than –70 dBm for reliable operation) |
| 19 | ☐ | Ping the device IP from your network — responds |

### UDP Motif Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 20 | ☐ | Send test motif from PC: echo '{"id":"test","repeat":1,"steps":[{"ch":"lra","type":"effect","effect":1,"dur_ms":500}]}' ¦ nc -u <DEVICE_IP> 9000 |
| 21 | ☐ | LRA produces a single “strong click” effect |
| 22 | ☐ | Send servo test: echo '{"id":"servo_test","repeat":1,"steps":[{"ch":"servo","type":"pos","pos_us":1000,"dur_ms":500},{"ch":"servo","type":"center","dur_ms":0}]}' ¦ nc -u <DEVICE_IP> 9000 |
| 23 | ☐ | Servo moves to position and returns to center |
| 24 | ☐ | Send combined dual-channel motif — both channels respond in sequence |
| 25 | ☐ | Repeat count works (repeat:3 plays the sequence three times) |

### Safety Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 26 | ☐ | Idle timeout: after 5 seconds of no motifs, servo auto-centers |
| 27 | ☐ | Watchdog: block the loop (temporary test) — device resets after 8 seconds |
| 28 | ☐ | Low battery: if battery < 3.3 V, serial warning appears and motors stop |
| 29 | ☐ | Wi-Fi reconnect: disconnect router, wait 15 s, reconnect — device re-establishes connection |

### Enclosure Fit Checks

| # | Pass | Test Description |
| --- | --- | --- |
| 30 | ☐ | Lid snaps on and off without excessive force |
| 31 | ☐ | USB-C cable plugs in through the port cutout (both XIAO and TP4056 if used) |
| 32 | ☐ | LRA vibration is clearly felt through the lid LRA window |
| 33 | ☐ | No component rattling when shaking the assembled puck |
| 34 | ☐ | Wrist strap threads through both strap slots and secures comfortably |
| 35 | ☐ | Overall weight < 70 g (target ~55 g) |

# 6. JSON Motif Schema Reference

The Wrist-Puck accepts haptic motifs as JSON payloads sent via UDP on port 9000. Each motif is a self-contained sequence of channel actions.

### Base Schema

{
  "id": "motif_name",
  "repeat": 1,
  "steps": [
    {
      "ch": "lra",
      "type": "effect",
      "effect": 47,
      "dur_ms": 200
    }
  ]
}

### Top-Level Fields

| Field | Type | Range | Description |
| --- | --- | --- | --- |
| id | string | — | Human-readable name for the motif |
| repeat | int | 1–10 | Number of times to play the full sequence |
| steps | array | — | Ordered list of channel actions |

### Step Fields — LRA Channel ("ch": "lra")

| Type Value | Additional Fields | Description |
| --- | --- | --- |
| "effect" | "effect": 1–123 | Play a DRV2605L library effect. See TI DRV2605L datasheet Table 11.2 for the full waveform library. |
| "rtp" | "amplitude": 0–255 | Real-Time Playback mode. Continuous vibration at specified intensity. |
| "stop" | — | Stop LRA output immediately. |

### Step Fields — Servo Channel ("ch": "servo")

| Type Value | Additional Fields | Description |
| --- | --- | --- |
| "pos" | "pos_us": 750–2250 | Move to position specified in microseconds pulse width. |
| "center" | — | Return to center position (1500 μs). |
| "sweep" | — | Execute a full left–right–center sweep test. |

### Common Step Field

| Field | Type | Range | Description |
| --- | --- | --- | --- |
| dur_ms | int | 0–5000 | Dwell time in milliseconds after executing this step before the next step begins. |

### Example Motifs

Motif 1 — “Rough Surface Bump”

{
  "id": "rough_bump",
  "repeat": 1,
  "steps": [
    {"ch": "lra",   "type": "effect", "effect": 47, "dur_ms": 100},
    {"ch": "servo", "type": "pos",    "pos_us": 1100, "dur_ms": 150},
    {"ch": "lra",   "type": "effect", "effect": 52, "dur_ms": 80},
    {"ch": "servo", "type": "center", "dur_ms": 0}
  ]
}

Motif 2 — “Weight Shift Left”

{
  "id": "weight_left",
  "repeat": 1,
  "steps": [
    {"ch": "servo", "type": "pos",  "pos_us": 800,   "dur_ms": 400},
    {"ch": "lra",   "type": "rtp",  "amplitude": 60, "dur_ms": 300},
    {"ch": "lra",   "type": "stop", "dur_ms": 0},
    {"ch": "servo", "type": "center", "dur_ms": 0}
  ]
}

Motif 3 — “Heartbeat”

{
  "id": "heartbeat",
  "repeat": 3,
  "steps": [
    {"ch": "lra",   "type": "effect", "effect": 1, "dur_ms": 120},
    {"ch": "lra",   "type": "effect", "effect": 1, "dur_ms": 300},
    {"ch": "servo", "type": "pos",    "pos_us": 1350, "dur_ms": 100},
    {"ch": "servo", "type": "center", "dur_ms": 400}
  ]
}

# 7. Troubleshooting

| # | Problem | Likely Cause | Solution |
| --- | --- | --- | --- |
| 1 | DRV2605L not found on I2C | SDA/SCL wires swapped, missing power, or missing pull-up resistors | Check SDA/SCL wiring matches pin table. Verify 3.3 V power to breakout. Add external 4.7 kΩ pull-ups if board lacks them. Try Wire.begin(SDA_PIN, SCL_PIN) with explicit pins. |
| 2 | LRA doesn’t vibrate | Wrong polarity on OUT pads, auto-cal failure, or LRA not making contact | Verify LRA wires on OUT+/OUT–. Check auto-cal passed in serial output. Try manual RTP mode: set amplitude to 200 and listen for vibration. |
| 3 | Servo jitters or doesn’t move | Insufficient voltage, wrong PWM pin, or electrical noise | Verify 5 V power (servo needs 4.8 V+). Confirm PWM pin assignment in firmware. Try a different GPIO. Add a 100 μF capacitor across servo VCC/GND. |
| 4 | Wi-Fi won’t connect | Wrong credentials, 5 GHz network, or antenna issue | Double-check SSID and password (case-sensitive). Verify the network is 2.4 GHz — ESP32-C3 does not support 5 GHz. Check onboard antenna is not obstructed by wiring. |
| 5 | Battery reads 0 V | Voltage divider wiring error, wrong ADC pin, or cold solder joint | Measure the divider midpoint with a multimeter (should read ~half of VBAT). Verify ADC pin matches firmware config. Re-solder joints. |
| 6 | Watchdog resets loop | A motif sequence total duration exceeds 8 seconds | Reduce step count or individual dur_ms values. Break long motifs into multiple shorter motifs with repeat. |
| 7 | Enclosure too tight | Printer dimensional accuracy or warping | Increase cavity dimensions by 0.5 mm in OpenSCAD parameters (CLEARANCE variable). Re-slice and re-print. Sand edges lightly if minor. |
| 8 | LRA too quiet through lid | Lid membrane too thick or LRA not seated against skin side | Enlarge the LRA window opening in OpenSCAD. Reduce LID_T to 1.0 mm. Ensure LRA is press-fit firmly into the floor pocket. |

# 8. Future Enhancements

The following features are planned or under consideration for future revisions of the Wrist-Puck platform:

| # | Enhancement | Description |
| --- | --- | --- |
| 1 | BLE MIDI Support | Add BLE MIDI input for musical haptic performances — map MIDI note-on/off events to LRA effects and servo positions in real time. |
| 2 | Multi-Node Mesh Networking | Implement ESP-NOW peer-to-peer mesh for a full-body haptic suit. Synchronize motif playback across wrist, chest, and ankle nodes with sub-10 ms latency. |
| 3 | IMU Integration | Add an MPU-6050 6-axis IMU for motion-reactive haptics. Enable gesture-triggered feedback and orientation-aware weight-shift patterns. |
| 4 | Custom PCB | Design a purpose-built PCB to replace the perfboard junction. Integrate the DRV2605L, voltage divider, servo header, and LiPo connector on a single 25 mm circular board. |
| 5 | OTA Firmware Updates | Implement over-the-air updates via the ArduinoOTA library. Eliminates the need to open the enclosure for firmware changes. |
| 6 | Web-Based Motif Designer | Host a local web UI on the ESP32-C3 (AP mode + HTTP server). Provide a drag-and-drop timeline editor for composing and testing motif sequences directly from a browser. |

Wrist-Puck Dual-Channel Haptic Node — Complete Build Guide  |  Rev 1.0  |  May 2026  |  Sean (Whitehorse, YT)

This document is provided as-is for personal and educational use.